Author(s): Jeng, SW (Jeng, Sheng-Wen); Tsai, WH (Tsai, Wen-Hsiang)

Title: Analytic image unwarping by a systematic calibration method for omni-directional cameras with hyperbolic-shaped mirrors

Source: IMAGE AND VISION COMPUTING, 26 (5): 690-701 MAY 1 2008

Language: English

Document Type: Article

Author Keywords: omni-directional camera; single-viewpoint (SVP) constraint; image unwarping; hyperbolic mirror; hypercatadioptric camera; camera calibration; analytic solution

Abstract: Unwarping an orimi-directional image into a perspective-view one is easy for a single-viewpoint (SVP) designed catadioptric omnidirectional camera. But misalignment between the components (such as the mirror and the lens) of this kind of camera creates multiple viewpoints and distorts the unwarped image if the SVP constraint is assumed. The SVP constraint is relaxed in this study and a systematic method is proposed to derive a set of new and general analytic equations for unwarping images taken from an omni-directional camera with a hyperbolic-shaped mirror (called a hypercatadioptric camera). The derivation is made possible by careful investigation on the system configuration and precise calibration of involved system parameters. As a verification of the correctness of the derived equations, some of the system parameters are adjusted to fit the SVP constraint, and unwarped images using the resulting simplified camera model are shown to be of no difference from those obtained by a method based on the SVP model. The generality of the proposed method so has extended the image-unwarping capability of the existing methods for the hypercatadioptric camera to tolerate lens/mirror assembly imprecision, which is difficult to overcome in most real applications. Some experimental results of image unwarping are also included to show the effectiveness of the proposed method. (C) 2007 Elsevier B.V. All rights reserved.

Addresses: [Jeng, Sheng-Wen; Tsai, Wen-Hsiang] Natl Chiao Tung Univ, Dept Comp Sci, Hsinchu 300, Taiwan; [Tsai, Wen-Hsiang] Asia Univ, Dept Comp Sci & Informat Engn, Taichung 41354, Taiwan

Reprint Address: Jeng, SW, Natl Chiao Tung Univ, Dept Comp Sci, 1001 Ta Hsueh Rd, Hsinchu 300, Taiwan.

E-mail Address: sunny@itri.org.tw; whtsai@cis.nctu

Cited References: ALIAGA D, 2001, P IEEE INT C COMP VI, V1, P127. BAKER S, 1999, INT J COMPUT VISION, V35, P175. FABRIZIO J, 2002, P IEEE WORKSH OMN VI, P45. GEYER C, 1999, P 7 INT C COMP VIS, V1, P398. GEYER C, 2002, IEEE T PATTERN ANAL, V24, P687. JENG SW, 2004, P 2004 IEEE INT C NE, P204. KANG SB, 2000, P IEEE C COMP VIS PA, V1, P201. NAYAR SK, 1997, PROC CVPR IEEE, P482. ONOE Y, 1998, P 14 INT C PATT REC, V1, P588. SALVI J, 2002, PATTERN RECOGN, V35, P1617. SWAMINATHAN R, 2001, P IEEE INT C COMP VI, V2, P2. TSAI RY, 1986, P IEEE C COMP VIS PA, P364. YAMAZAWA K, 1993, P INT C INT ROB SYST, P1029.

Cited Reference Count: 13

Times Cited: 1

Publisher: ELSEVIER SCIENCE BV

Publisher Address: PO BOX 211, 1000 AE AMSTERDAM, NETHERLANDS

ISSN: 0262-8856

DOI: 10.1016/j.imavis.2007.08.005

29-char Source Abbrev.: IMAGE VISION COMPUT

ISO Source Abbrev.: Image Vis. Comput.

Source Item Page Count: 12

Subject Category: Computer Science, Artificial Intelligence; Computer Science, Software Engineering; Computer Science, Theory & Methods; Engineering, Electrical & Electronic; Optics

ISI Document Delivery No.: 284DU