Accession number:20095312593714
Title: Variation of lateral thickness techniques in sol lateral high voltage transistors
Authors: Guo, Yufeng (1); Wang, Zhigong (1); Sheu, Gene (3)
Author affiliation:(1) Institute of RF- and OE-ICs, Southeast University, Nanjing, Jangsu, 210096, China; (2) School of Electronic Science and Engineering, Nanjing University of Posts and Telecommunications, Nanjing, Jiangsu, 210003, China; (3) Department of Computer Science and Information Engineering, Asia University, Taichung, Taiwan
Corresponding author:Guo, Y.
(yfguo@njupt.edu.cn)
 
Source title:2009 International Conference on Communications, Circuits and Systems, ICCCAS 2009
Abbreviated source title:Int. Conf. Commun., Circuits Syst., ICCCAS
Monograph title:2009 International Conference on Communications, Circuits and Systems, ICCCAS 2009
Issue date:2009
Publication year:2009
Pages:611-613
Article number:5250456
Language:English
ISBN-13:9781424448883
Document type:Conference article (CA)
Conference name:2009 International Conference on Communications, Circuits and Systems, ICCCAS 2009
Conference date:July 23, 2009 - July 25, 2009
Conference location:Milpitas, CA, United states
Conference code:78163
Sponsor:University of Electronic Science and Technology of China, UESTC; University of California
Publisher:IEEE Computer Society, 445 Hoes Lane - P.O.Box 1331, Piscataway, NJ 08855-1331, United States
Abstract:A novel variation of Lateral Thickness(VLT) technique is proposed to bring a uniform surface electric field of SOl lateral high voltage devices. Comparing to the conventional RESURF device, the linear thickness of drift region increases the breakdown voltage by 40% while decreasing the drift resistance by 50%. Furthermore, single- or two-step drift thickness can be adopted to reduce fabrication difficulties when higher breakdown voltage and lower drift resistance are maintained. &copy; 2009 IEEE.
Number of references:8
Main heading:Sols
Controlled terms: Electric breakdown - Electric fields
Uncontrolled terms:Breakdown voltage - Drift regions - Drift resistance - High voltage devices - High voltage transistor - Surface electric fields
Classification code:701.1 Electricity: Basic Concepts and Phenomena - 801.3 Colloid Chemistry - 804 Chemical Products Generally
Database:Compendex
Compilation and indexing terms, Copyright 2009 Elsevier Inc.